iW-SA30  SIP  Speaker

| IW - SA30 is based on the VOIP phone Fanvil technology industry research and development of user requirements specifically for a SIP speakers products, voice transmission adopts the standard of IP/RTP protocol, it has inherited good bearing phone compatibility good advantages, can be perfectly compatible with the present all mainstream IPPBX/softswitch based on SIP/IMS platform, such as Asterisk, Broadsoft, 3 cx, Elastix, rapid deployment device provides a convenient for the user. Function Highlights - Compatible with standard SIP (RFC3261) protocol and main IPPBX/IMS platform - All-in-one wall-mounted design, integrated with network audio codec, digital amplifier amplifier and speaker - Built-in high fidelity speaker - Online adjust volume output - The built-in audio output interface to connect external active speakers - Standard RJ45 port, the Ethernet front-end ports where you can access, support across different network segment and routing - Adaptive output power according to the power supply voltage | Applications - Business office, hotel, shopping center - Education schools,the gym - public places factory, museum - Transportation airport, parking lot |
| --- | --- |

Basic Performance

- 2 SIP lines

- POE enabled(Power Over Ethernet)

- Wall-mounted installation

- A built-in audio output to connect external speaker

- Support external power supply

- Industry certification: CE/FCC

Audio Features

- Narrowband speech coding: G.711a/u, G.723.1, G.726-32K, G.729AB

- Wideband speech coding: G.722

- voice activation detection/mute

- DTMF: With the audio (In-band), Out-of-band (RFC2833) / SIP INFO

Network/Security features

- Support PoE(802.3af)

- support the DNS and secondary DNS server

- Support VLAN

- Support SNTP client

- Support VPN L2TP / PPTP / IPSec

- Support SRTP

- Support web page HTTP / HTTPS

- Support Qos:802.1p/q, DSCP

- Support the MD5 authentication

- Support Web Filter

- DHCP / static / PPPoE

- STUN

Configuration and Management

- Via FTP/TFTP DHCP OPT66 / SIP/HTTP/HTTPS/PNP/TR069 automatically

- web management interface

- Web Packet capture (Web -based Packet - dump)

- Configuration export/import

- Software upgrade

- System logs (Syslog)

Physical Specifications

- WAN port：10/100BASE-Tx，Auto-MDIX，RJ-45

- LAN port：10/100BASE-Tx，Auto-MDIX，RJ-45

- PoE：802.3af(Class 3 - 6.49~12.95W)

- Main chip: Broadcom

- Output power: according to the power supply voltage automatic adaptation. A) PoE 8 w; B) the 12 v / 1.5 A, 10 w. C) 18 v / 3 a 20 w; D) 24 v / 3 a 30 w

- Horn frequency response: 75 hz - 20 KHZ

- Sound pressure (SPL 12/1 meters) : 100 db

- Power supply: 12 v / 1 a DC or PoE

- Working temperature: - 10 ~ 50 ℃

- Storage temperature: - 10 ~ 50 ℃

- Relative humidity: 20 ~ 80%

- Package weight: 3 kg

- Equipment size: 245 x162x182mm (W x H x L)

Above functions/features/specifications provide customer reference only, may change in the future, bearing without prior notification obligations, all functions/features/specifications must will be subject to the actual shipment of goods. If the customer about these functions/features/specifications have doubt, please contact distributors around the agent or bearing.

Check out more product information please visit bearing's official website: www.fanvil.com

Compatibility List